Exp 24 MATLAB CODE

clc; clear; close all;

% Parameters

N = 10000; Fs = 100000; fc = 10000;

SNR = 10;

% Generate Random Bits data = randi([0 1],1,N);

% BPSK Modulation symbols = 2*data - 1;

% Passband Transmission t = (0:N-1)/Fs; tx = symbols .* cos(2*pi*fc*t);

% Multipath Fading Channel h = [1 0.5 0.3]; faded_signal = filter(h,1,tx);

% Add AWGN Noise signal_power = mean(faded_signal.^2); noise_power = signal_power/(10^(SNR/10)); rx = faded_signal + sqrt(noise_power)*randn(size(faded_signal));

% Coherent Demodulation demod = rx .* cos(2*pi*fc*t);

% Low-Pass Filtering

[b,a] = butter(5,2000/(Fs/2),'low'); baseband = filtfilt(b,a,demod);

% Remove Initial Transient start = 100; detected = baseband(start:end) > 0; original = data(start:end);

L = min(length(detected),length(original)); detected = detected(1:L); original = original(1:L);

% BER Calculation errors = sum(original ~= detected);

BER = errors/L;

fprintf('SNR = %d dB\n',SNR); fprintf('Total Bits = %d\n',L); fprintf('Bit Errors = %d\n',errors); fprintf('Bit Error Rate (BER) = %.6f\n',BER);

% Plot Results

figure;

subplot(3,1,1); plot(t(1:500),tx(1:500)); title('Transmitted Passband BPSK Signal'); xlabel('Time (s)'); ylabel('Amplitude'); grid on;

subplot(3,1,2); plot(t(1:500),faded_signal(1:500)); title('Signal after Multipath Channel'); xlabel('Time (s)'); ylabel('Amplitude'); grid on;

subplot(3,1,3); plot(t(1:500),rx(1:500)); title('Received Signal with AWGN'); xlabel('Time (s)'); ylabel('Amplitude'); grid on;