Exp 26 MATLAB CODE

clc; clear; close all;

% Generator Matrix for Hamming (7,4) Code

- = [1 0 0 0 1 1 0;

  - 1 0 0 1 0 1;

0 0 1 0 0 1 1;

0 0 0 1 1 1 1];

% Parity Check Matrix

- = [1 1 0 1 1 0 0;

1 0 1 1 0 1 0;

  - 1 1 1 0 0 1];

% Input Message msg = [1 0 1 1];

% Encoding codeword = mod(msg * G, 2);

disp('Original Message:'); disp(msg);

disp('Encoded Codeword:'); disp(codeword);

% Introduce Single-Bit Error received = codeword; error_position = 3; received(error_position) = 1 - received(error_position);

disp('Received Codeword with Error:'); disp(received);

% Calculate Syndrome syndrome = mod(received * H', 2);

disp('Syndrome Vector:'); disp(syndrome);

% Error Detection and Correction error_position = 0;

for i = 1:7     if isequal(syndrome, H(:,i)')         error_position = i;         break;     end

end

if error_position ~= 0     received(error_position) = 1 - received(error_position);     disp('Single-Bit Error Detected and Corrected.'); else

disp('No Error Detected.'); end

disp('Corrected Codeword:'); disp(received);

disp('Recovered Message:'); disp(received(1:4));

% Verify Correction if isequal(received, codeword)     disp('Error Correction Successful.'); else     disp('Error Correction Failed.'); end

OUTPUT

Original Message:

1     0     1     1

Encoded Codeword:

1     0     1     1     0     1     0

Received Codeword with Error:

1     0     0     1     0     1     0

Syndrome Vector:

- 1     1

Single-Bit Error Detected and Corrected.

Corrected Codeword:

- 0     1     1     0     1     0

Recovered Message:

1     0     1     1

Error Correction Successful.

>>